# FS2000 IPC Protocol for Advanced Weather Interface

# (FSUIPC 1.90)

### By Pete Dowson, 8th May 2000

### Introduction

FS2000 provides a more capable weather system than FS98, yet weather programs are currently restricted to the subset of that system which is common to both. This is because the IPC variables used are those defined for FS98 in the global data area. Some minor additions were made to FSUIPC, using otherwise unused Global offsets, but these were by no means enough to allow full exploitation of FS2000 weather facilities.

This document outlines additions made to FSUIPC in version 1.40 to allow external weather programs to control and read all aspects of FS2000’s weather system.

### Basic method

In order to avoid the need for programs to be substantially altered, the new facilities are provided by utilising a range for global offsets which is well outside the FS98 global data area limits. Such offsets, used in the wrong version of FSUIPC or FS6IPC would probably cause an access violation and crash FS2000 (and FS98 too, of course). So, before attempting to use any of these facilities, the application should check that they are being supported. This will be indicated by the value

0xFADE	in the 16-bit word at offset 0x4D6.

If this value is NOT found there, then do not use these facilities!

The offsets accessed for advanced weather are all in the range 0x100000–0xFFFFFF, but very few of these are used. Unlike in normal IPC interchanges, the offset actually denotes a Command, not an Address, and the data being “read” or “written” to the offset can be thought as “parameters” for the command, and “results” when it is a read.

### Command Summary

The table on the next page lists all the weather specific command types and the weather aspects, showing how they apply. In addition to these there is a “clear all” command, and a “

All commands except the Clear command and list ones (Get and Set) and the Adventure weather ones, have an extra parameter build into the command word (offset): this is the altitude in metres (AMSL). This is used to determine which entry in the list of layers is being addressed: it’s a clearer, easier and less ambiguous system than trying to address layers by number.

The details applicable to each weather aspect are as follows (structures for these will follow in due course):

Cloud: Type, Bottom (AMSL), Top (AMSL), Deviation, Coverage, Turbulence, Precipitation type, Precipitation rate, Precipitation base, Icing.

Pressure: Pressure at MSL (QNH), Drift (N.B. I’m not certain of support for the latter in FS2000)

Temperature: Altitude (AMSL), Day temperature, Night variation, Dew point

Visibility: Bottom (AMSL), Top (AMSL), Visibility

Wind: Upper Altitude (AMSL), Speed, Direction (TRUE), Gust speed, Turbulence, Windshear

Although FS2000 uses floating point for many of these things internally, FSUIPC keeps to the use of 16-bit words, with the same units, on the whole, as FS98. For instance, all altitudes are in Metres, up to 65500 of them (allowing room for 35 metres of negative altitude, not really needed for weather data), Temperatures are in degrees C (not x 256), Pressure is in millibars x 16 (hectopascals x 16 if you like), Visibility is in Statute Miles x 100, and Direction is always in degrees TRUE, but still in the 65536 == 360 notation.

The differences are: Surface wind directions are degrees TRUE and their altitudes are AMSL, to match FS2000. Gust speed will be the upper speed (FSUIPC will handle the current FS2000 problem till it is fixed). Coverage will be in Octas (0–8). Turbulence, Shear and Icing will all use FS2000’s scales for these things.

| ## Command | Cloud | Pressure | Temperature | Visibility | Wind |
| --- | --- | --- | --- | --- | --- |
| GetAt (use READ) | Reads cloud layer details at or above specified AMSL | #### Not applicable | Reads interpolated temperature details at specified AMSL | Reads visibility layer details at specified AMSL | Reads wind layer details at specified AMSL |
| Get (use READ) | Reads complete cloud layers list | Reads pressure details | Reads complete temperature layers list | Reads complete visibility layers list | Reads complete wind layers list |
| Clear (use WRITE, but with no data) | Clears all cloud layers | Sets standard pressure (1013.2 mb) | Sets FS2K default temperatures | Sets FS2K default visibility | Clears all wind layers |
| Set (use WRITE) | Sets a complete new list of cloud layers | #### Not applicable | Sets a complete new list of temperature layers | Sets a complete new list of visibility layers | Sets a complete new list of wind layers |
| AddAt (use WRITE) | Inserts specified cloud layer, adjusting altitudes if there’s an overlap. | #### Not applicable | Inserts temperature level, adjusting altitudes if there’s an overlap. | Inserts visibility layer, adjusting altitudes if there’s an overlap. | Inserts wind layer, adjusting altitudes if there’s an overlap. |
| RemoveAt (use WRITE, but with no data) | Removes the cloud layer operating at or above the given AMSL | #### Not applicable | Removes the temperature setting operative at the given AMSL or above | Removes the visibility layer operative at the given AMSL or above | Removes the wind layer operative at the given AMSL, or above |
| ModifyAt (use WRITE) | Modifies cloud attributes in the layer operating at or above the given AMSL without impinging on any other layers. | Sets pressure details | Modifies temperatures in the layer operating at or above the given AMSL without changing layer levels. | Modifies visibility in the layer operating at or above the given AMSL without changing layer levels. | Modifies wind attributes in the layer operating at or above the given AMSL without changing layer levels. |

The C include file included with this document defines the actual offsets corresponding to each command, and the structures of the weather data which is read or written. The WEATHERSET.EXE program included in the FSUIPC package uses ALL of the above commands. Experiment. You can use the Logging in FSUIPC to see exactly what is going on.

Enjoy!